布里渊散射是入射光子与介质中热激发的声子相互作用产生的非弹性散射。流体的布里渊散射光谱通常由中心瑞利峰和对称分布的斯托克斯、反斯托克斯双峰构成。其中，瑞利峰来源于非传播的局部熵涨落，布里渊双峰来源于传播的绝热压力涨落（声波）。布里渊双峰相对中心瑞利峰的频移正比与介质中的绝热声速，其线宽则与声吸收系数相关；而中心瑞利峰与布里渊双峰的积分强度比称为Landau-Placzek比，由流体的热容比等热力学参数决定。因此，布里渊散射已成为探测流体声学与热力学参数的重要非接触手段，在海洋遥感、生物医学成像和流体力学诊断等领域均有应用。

由于流体的布里渊散射强度很弱、且频移约在GHz量级，因此通常使用法布里-珀罗干涉仪（FPI）以及光电倍增管（PMT）对光谱进行扫描，FPI主要由两面高反射率的平行镜片组成，透射光由于多光束干涉形成极窄的透射峰，通过调整两个镜片之间气压、距离进行调谐，从而实现光谱的扫描。此外，对于Landau-Placzek比的测量，对液体的纯度有很高的要求，否则杂质的散射会导致中心峰信号的增强。早在1965年，Benedek和Greytak就利用氦氖激光器和气压扫描式FPI对水和甲苯中的声波的声速与寿命进行了测量。1966年，Cummins和Gammon系统研究了液体中的布里渊散射，且重点研究并测量了常见液体的Landau-Placzek比，指出经典公式忽略了热力学性质的色散，使用超声速压缩率进行修正，验证了11种常见液体的修正后的比值与实验吻合更好。1967年，O'Connor和Schlupf专门研究了水中的布里渊散射，给出了0–50°C范围内水的声速以及Landau-Placzek比随温度的变化曲线，指出经典的朗道比值近似存在较大的误差，须使用精确表达式；实验中4°C时LP比趋近于零，与热膨胀系数为零的物理事实一致。2002年，Edward Fry等人设计了喇叭形的水槽减少杂散光，使用532nm的脉冲激光以及压电扫描式FPI测量背向散射，给出了1–35°C范围内水的布里渊线宽与温度的关系，指出测量的线宽随温度降低而显著增大：从10°C的750 MHz到1°C的1.4 GHz。文章分析认为线宽由体粘滞、剪切粘滞、热导率和比热共同决定，而低温下体粘滞的贡献急剧增加，并提出线宽测量可作为独立的温度反演手段。2017年，Zykova等人利用3+3通串联FPI，在249–365 K宽温度范围内以0.1 GHz的高光谱分辨率重新测量了水的布里渊散射光谱。此外，Hirschberg等（1984）、Emery和Fry（1997）、Fry等（1997）以及Schorstein等（2008）也基于布里渊光谱的测量对布里渊激光雷达做了研究，包括声速与温度的遥感测量、精度限制分析以及基于光纤放大器的远程水温测量等。

在国内，1983年西北大学的雷印生等人也使用了气压扫描式FPI测量了水的布里渊散射光谱。2013西安应用光学研究所的Li Tao等人基于F-P标准具设计了一套用于气流瑞利-布里渊散射光谱测量的成像式FPI，并且分析讨论了成像式干涉仪的分辨本领、自由光谱范围以及视场等。在该系统中，散射光通过标准具在ICCD上成像形成同心圆环，可以一次性得到光谱信息，相比扫描式FPI大幅缩短了探测时间。F-P标准具可以认为是一种腔长固定的FPI，光以不同角度射入标准具时，满足共振条件的波长不同，收集到的干涉图样实际上可以认为是入射光的光谱函数与标准具的透过函数积分得到的结果。2016年，华中科技大学的Bo Zhou等人使用F-P标准具结合ICCD对水的布里渊散射进行测量。针对ICCD探测图样的数据处理，作者提出频域上的卷积拟合方法，将基于半径-频率二维积分的拟合降为基于频率一维卷积的拟合，缩短了计算时间；通过拟合，实验在实验室条件下达到了0.1°C温度精度和0.5‰盐度精度，为快速遥感探测提供了可行方案。2025年，上海光机所的Ye Bo等人设计了一套基于VIPA（虚拟成像相位阵列）标准具的离轴布里渊光谱系统，研究了液体中布里渊散射强度的角度依赖性。他们以瑞利散射为参考，测得水的朗道比为0.015，比以往报道更接近理论值0.01；作者通过实验分析认为瑞利与布里渊散射具有相同的角度依赖关系，为复杂几何下的光谱分析提供了定量框架。

在前人的基础上，本课题拟搭建一套基于F-P标准具的布里渊光谱测量系统。相较于扫描式FPI，标准具无需机械扫描，可一次性得到包含完整光谱信息的图样。数据处理时需将干涉图样上的径向分布的空间信息反演得到频域上的光谱信息，同时，标准具的透射函数会引入仪器展宽的影响。此外，散射光强极弱同样也是需要克服的困难之一。本课题希望通过搭建的光学系统测量水的布里渊散射光谱，采用数值方法提取光谱特征，建立声速、声吸收系数等声学参数与温度的关系曲线，最后验证、评估系统的测量精度、灵敏度及时间分辨能力，探索复杂流体诊断的布里渊光谱方案。

主要参考文献：

[1] BENEDEK G, GREYTAK T. Brillouin scattering in liquids[J]. Proceedings of the IEEE, 1965, 53(10): 1623-1629.

[2] CUMMINS H Z, GAMMON R W. Rayleigh and Brillouin scattering in liquids: The Landau—Placzek ratio[J]. The Journal of Chemical Physics, 1966, 44(7): 2785-2796.

[3] MOUNTAIN R D. Spectral distribution of scattered light in a simple fluid[J]. Reviews of Modern Physics, 1966, 38(1): 205-214.

[4] O'CONNOR C L, SCHLUPF J P. Brillouin scattering in water: The Landau—Placzek ratio[J]. The Journal of Chemical Physics, 1967, 47(1): 31-38.

[5] HIRSCHBERG J G, BYRNE J D, WOUTERS A W, et al. Speed of sound and temperature in the ocean by Brillouin scattering[J]. Applied Optics, 1984, 23(15): 2624-2628.

[6] EMERY Y, FRY E S. Laboratory development of a lidar for measurement of sound velocity in the ocean using Brillouin scattering[C]//Proc. SPIE 2963, Ocean Optics XIII, 1997: 210-215.

[7] FRY E S, EMERY Y, QUAN X, et al. Accuracy limitations on Brillouin lidar measurements of temperature and sound speed in the ocean[J]. Applied Optics, 1997, 36(27): 6887-6894.

[8] BŁACHOWICZ T. The use of pressure controlled Fabry-Pérot interferometer with linear scanning of data for Brillouin-type experiments[J]. Review of Scientific Instruments, 2000, 71(8): 2988-2991.

[9] FRY E S, KATZ J, LIU D, et al. Temperature dependence of the Brillouin linewidth in water[J]. Journal of Modern Optics, 2002, 49(3-4): 411-418.

[10] SCHORSTEIN K, POPESCU A, GÖBEL M, et al. Remote water temperature measurements based on Brillouin scattering with a frequency doubled pulsed Yb:doped fiber amplifier[J]. Sensors, 2008, 8(9): 5820-5831.

[11] LI T, LI G P, WU L, et al. Analysis and design of imaging Fabry-Perot interferometers for measurement of Rayleigh-Brillouin scattering spectra in gas flows[C]//Proc. SPIE 8910, International Symposium on Photoelectronic Detection and Imaging 2013: Imaging Spectrometer Technologies and Applications, 2013: 89101E.

[12] ZHOU B, FAN Q, MA Y, et al. Experimental analysis on the rapid measurement of a high precision Brillouin scattering spectrum in water using a Fabry–Pérot etalon[J]. Laser Physics Letters, 2016, 13(5): 055701.

[13] ZYKOVA V A, KARPEGINA Y A, MALINOVSKY V K, et al. Temperature dependence of the Landau-Placzek ratio in liquid water[J]. Physical Review E, 2017, 96(4): 042608.

[14] YE B, YAO S, QI Y, et al. Angle dependence of Brillouin scattering intensity in liquids[J]. Optics Letters, 2025, 50(8): 2502-2505

[15]雷印生, 王诺, 张镇西. 液体中He-Ne激光布里渊散射的实验研究[J]. 物理, 1983, 12(7): 415-417.